# DAY3_ELECTRONICS_QUIZE

# Questions

- What is the typical forward voltage drop of a silicon diode when conducting?
a) about 0.1 V
b) about 0.7 V
c) about 2.0 V
d) about 5.0 V

- What is the main difference between an NPN BJT and an N-channel MOSFET in how they are driven?
a) BJT is voltage-driven at the base, MOSFET is current-driven at the gate
b) BJT is current-driven at the base, MOSFET is voltage-driven at the gate
c) Both are driven by voltage only
d) Both are driven by current only

- For an ideal inverting op-amp configuration with input resistor Rin and feedback resistor Rf, what is the closed-loop voltage gain Av?
a) Av=RinRf
b) Av=-RfRin
c) Av=RfRin
d) Av=-RinRf

- What is the cutoff frequency fc of a simple RC low-pass filter?
a) fc=2πRC
b) fc=1RC
c) fc=12πRC
d) fc=RC

- What does ESR stand for in the context of capacitors?
a) Equivalent series resistance
b) Electric series reactance
c) Effective storage rating
d) External shunt resistor

- What is the purpose of an AC coupling (series) capacitor between two circuit stages?
a) To pass DC and block AC
b) To block DC and pass AC
c) To increase DC offset
d) To store energy for long-term supply

- Which of the following is a primary benefit of using a continuous ground plane on a PCB?
a) Increasing trace impedance for signal lines
b) Providing a low-impedance return path and shielding, reducing EMI and noise
c) Making the board lighter
d) Eliminating the need for decoupling capacitors

- What is the purpose of thermal vias placed under a power IC or LED?
a) To electrically isolate the component from the board
b) To route signals to inner layers only
c) To transfer heat from the top-side pad to inner or bottom copper planes to improve thermal dissipation
d) To increase solder joint mechanical flexibility

- How is a power ratio expressed in decibels (dB)?
a) dB = 20log⁡10P2P1
b) dB = 10log⁡10P2P1
c) dB = log⁡10P2P1
d) dB = 2log⁡10P2P1

- What typically happens when an op-amp output saturates against its supply rail?
a) The op-amp continues linear amplification beyond the rails with no effect
b) The output is limited near the supply rail and the op-amp cannot increase output further; the input difference may remain nonzero (loss of linear operation)
c) Saturation causes the op-amp to invert its inputs automatically
d) Saturation improves bandwidth and reduces distortion

# Answers

- Answer: b) about 0.7 V — Correct
Explanation: A typical silicon diode forward drop when conducting around moderate current is ≈0.6–0.8 V (commonly cited as 0.7 V). Actual Vf depends on diode type and current (Schottky: lower, power diodes: similar or higher).

- Answer: b) BJT is current-driven at the base, MOSFET is voltage-driven at the gate — Correct
Explanation: BJT input requires base current to bias collector-emitter current (current-controlled device). MOSFET gate is insulated (capacitive) and is driven by gate voltage to control channel conduction (voltage-controlled device); gate draws negligible steady DC current (only charging/discharging currents during transitions).

- Answer: b) Av=-RfRin — Correct
Explanation: In an ideal inverting op-amp (infinite open-loop gain, input at inverting node with Rin and feedback Rf), closed-loop gain is Av=-Rf/Rin. The minus sign indicates a 180° phase inversion.

- Answer: c) fc=12πRC — Correct
Explanation: Cutoff (–3 dB) frequency of a simple RC low-pass: fc=12πRC. Below fc the output passes with little attenuation; above fc the output falls at −20 dB/decade.

- Answer: a) Equivalent series resistance — Correct
Explanation: ESR = Equivalent Series Resistance, the resistive loss inside a capacitor that causes dissipation, heating and reduced performance at high ripple currents.

- Answer: b) To block DC and pass AC — Correct
Explanation: An AC-coupling (series) capacitor removes DC offset between stages while passing the AC component. It forms a high-pass filter with the input impedance of the following stage; cutoff fc≈1/(2πRinCcouple).

- Answer: b) Providing a low-impedance return path and shielding, reducing EMI and noise — Correct
Explanation: A continuous ground plane gives a short return path for signals, lowers ground impedance, provides shielding, reduces loop area (lower EMI), and improves signal integrity.

- Answer: c) To transfer heat from the top-side pad to inner or bottom copper planes to improve thermal dissipation — Correct
Explanation: Thermal vias (plated thru-holes under power ICs or LEDs) conduct heat into internal/bottom copper planes or heatsinks, reducing junction temperature and improving reliability. They must be filled/plated appropriately for best thermal performance.

- Answer: b) dB = 10log⁡10P2P1 — Correct
Explanation: Power ratio in decibels uses 10log⁡10 because dB is defined on a power basis. (For voltage ratios across the same impedance use 20log⁡10(V2/V1).)

- Answer: b) The output is limited near the supply rail and the op-amp cannot increase output further; the input difference may remain nonzero (loss of linear operation) — Correct
Explanation: When an op-amp saturates at a rail, the output is clamped near the supply and the internal amplification cannot move the output further; the differential input can remain nonzero and the amplifier is no longer operating in its linear (closed-loop) region. Recovery requires changing inputs or using methods to prevent saturation (e.g., feedback design, output stage margin).