# DAY4_ELECTRONICS_QUIZE

# Questions

- What is the typical forward voltage drop of a Schottky diode when conducting (compared to a silicon diode)?
a) about 0.1 V
b) about 0.3–0.5 V
c) about 0.7 V
d) about 2.0 V

- What is the unit and meaning of an op-amp’s slew rate?
a) V/Ω — voltage per resistance, how fast output changes with load
b) V/s — volts per second, DC output drift rate
c) V/µs — volts per microsecond, maximum rate of change of the output voltage
d) A/V — amps per volt, input transconductance

- For an ideal LC resonant tank, what is the resonance frequency f0?
a) f0=1RC
b) f0=12πLC
c) f0=2πLC
d) f0=LC

- What is the primary purpose of an RC or RCD snubber placed across a switching transistor or transformer winding?
a) To increase switching speed by adding capacitance
b) To filter low-frequency noise only
c) To absorb/limit voltage spikes and reduce dv/dt, protecting the switch and reducing EMI
d) To act as the main energy storage for the converter

- If duty cycle D of a PWM waveform is defined by on-time ton and period T, which formula is correct?
a) D=ton⋅T
b) D=tonT
c) D=Tton
d) D=ton+T

- What is the main function of a heatsink attached to a power component?
a) To electrically insulate the component from the board
b) To increase thermal resistance and keep the component hotter
c) To conduct heat away from the component to ambient, lowering junction temperature and improving reliability
d) To store thermal energy like a capacitor

- Why is the characteristic impedance Z0 of a transmission line important for high-speed signals?
a) It determines the DC current capacity of the line only
b) Matching Z0 between source, line, and load minimizes reflections and preserves signal integrity
c) It sets the board’s thermal profile
d) It is only relevant for power traces, not signals

- What is the typical use of a Zener diode on a power rail?
a) To generate clock signals for digital logic
b) To clamp or regulate the voltage (reference/clamp) when operated in its breakdown region
c) To act as the primary current-carrying device in power conversion
d) To increase supply voltage under load

- What does the Nyquist sampling theorem state for sampling a continuous signal?
a) Sampling frequency must equal the signal frequency for correct reconstruction
b) Sampling frequency fs must be strictly less than twice the maximum signal frequency fmax
c) Sampling frequency fs must be at least twice the highest frequency component fmax of the signal (fs≥2fmax) to avoid aliasing
d) Nyquist only applies to DC signals

- Which instrument is best for measuring and plotting the magnitude and phase frequency response of a two-port network (S-parameters) over a wide frequency range?
a) Digital multimeter (DMM)
b) Oscilloscope
c) Vector network analyzer (VNA)
d) Power supply

# Answers

- Answer: b) — Correct
Explanation: A Schottky diode typical forward drop is lower than a silicon diode, roughly 0.3–0.5 V depending on current and device. Silicon junction diodes are around 0.6–0.8 V.

- Answer: b) — Correct
Explanation: Slew rate is in units of volts per microsecond (V/µs). It defines the maximum rate of change of the output voltage of an op-amp (how fast the output can change), limiting the largest amplitude of high-frequency signals the amplifier can reproduce without distortion.

- Answer: b) — Correct
Explanation: Resonant frequency of an LC tank: f0=12πLC. Option b is the correct expression.

- Answer: c) — Correct answer: c)
Explanation: An RC or RCD snubber across a switching device or transformer winding is used to absorb or limit voltage spikes and reduce dv/dt, protecting the switch and reducing EMI. It’s not primarily for increasing switching speed or acting as the converter’s main energy store. Typical snubber designs: RC (damps ringing), RCD (clamps and dissipates energy), or RC+diode variants depending on topology.

- Answer: b) — Correct
Explanation: Duty cycle D is defined as D=ton/T (on-time divided by period).

- Answer: c) — Correct
Explanation: A heatsink conducts heat away from the component to ambient (via conduction/convection/radiation), lowering junction temperature and improving reliability.

- Answer: b) — Correct
Explanation: Characteristic impedance Z0 matters because matching source, line and load to Z0 minimizes reflections and preserves signal integrity at high speed.

- Answer: b) — Correct
Explanation: Zener diodes are commonly used to clamp or regulate voltage when biased in their breakdown region (reference/clamp).

- Answer: c) — Correct
Explanation: Nyquist: sampling frequency must be at least twice the highest signal frequency component: fs≥2fmax to avoid aliasing.

- Answer: c) — Correct
Explanation: Vector Network Analyzer (VNA) is the right instrument to measure magnitude and phase (S-parameters) over a wide frequency range.