# DAY5_ELECTRONICS_QUIZE

# Questions

- What does a Thevenin equivalent circuit represent?
a) A single voltage source in series with an impedance
b) A current source in parallel with an impedance
c) A voltage divider and load combination only
d) A purely resistive replacement for reactive networks

- What does CMRR (common-mode rejection ratio) of a differential amplifier quantify?
a) The amplifier’s input impedance for differential signals
b) The maximum differential output swing before clipping
c) The amplifier’s ability to reject signals common to both inputs
d) The ratio of output noise to input offset voltage

- What is the main purpose of a common-mode choke on a power or signal line?
a) To pass DC while blocking AC equally for both lines
b) To act as a fuse for overcurrent protection
c) To provide a low-impedance path for return currents
d) To attenuate common-mode noise while allowing differential signals to pass

- What does a soft-start circuit in a DC-DC converter do?
a) Switches the converter off when temperature exceeds a threshold
b) Gradually ramps output voltage/current at startup to limit inrush and stress
c) Converts startup transient energy into heat for faster regulation
d) Bypasses the feedback loop during normal operation

- Which MOSFET parameter most directly limits switching speed and determines gate drive requirements?
a) Total gate charge (Qg) or gate capacitance
b) Drain-source breakdown voltage (Vds_max)
c) Body diode forward voltage (Vf)
d) Drain current rating (Id_max)

- Which description best matches thermal runaway in a semiconductor device?
a) A controlled increase in temperature to improve conduction
b) A thermal cycle that increases reliability over time
c) Temperature oscillation caused by ambient cooling variations
d) A positive feedback loop where rising temperature increases leakage/current, causing further heating

- What is a primary benefit of differential signaling (e.g., twisted pair, RS-485) over single-ended signaling?
a) Higher DC power delivery capability
b) Lower manufacturing cost due to fewer wires
c) Better immunity to external EMI and reduced ground-reference issues
d) Simpler receiver circuitry with no need for termination

- What is the purpose of a star ground (single-point ground) scheme on a PCB or system?
a) To connect all grounds in a daisy chain to reduce trace length
b) To route all subsystem grounds to a single central point to minimize ground loops
c) To intentionally split ground planes for signal isolation without interconnects
d) To route analog and digital grounds separately with no common connection

- What effect does sampling aperture jitter (timing uncertainty) have on an ADC measuring a high-frequency signal?
a) It creates amplitude error/noise proportional to the signal slew rate and jitter magnitude
b) It changes the ADC reference voltage and shifts all codes
c) It reduces the ADC input impedance at high frequencies
d) It increases the ADC resolution by averaging timing errors

- What does the input common-mode voltage range of an op-amp specify?
a) The maximum allowed differential input voltage magnitude
b) The required supply voltage margin for stable operation
c) The voltage range at which the output saturates regardless of input
d) The range of input voltages (relative to rails) over which the inputs behave correctly and the amplifier operates as specified

# Answers

- Correct answer: a) A single voltage source in series with an impedance
Explanation: A Thevenin equivalent replaces any linear two-terminal network by a single open-circuit voltage source Vth in series with a single impedance Zth (or resistance Rth for purely resistive). It is not limited to just a divider+load — it’s a general equivalent.

- Correct answer: c) The amplifier’s ability to reject signals common to both inputs 
Explanation: CMRR quantifies how well a differential amplifier rejects common-mode signals (same signal on both inputs). High CMRR means the amplifier amplifies the differential input while ignoring common noise.

- Correct answer: d) To attenuate common-mode noise while allowing differential signals to pass
Explanation: A common-mode choke has windings that present high impedance to common-mode currents (noise on both lines) while presenting low impedance to differential signals, so differential data passes but common-mode interference is attenuated.

- Correct answer: b) Gradually ramps output voltage/current at startup to limit inrush and stress 
Explanation: Soft-start limits inrush current and prevents large startup stress by ramping control (duty or reference) so the converter output rises slowly and components aren’t shocked.

- Correct answer: a) Total gate charge (Qg) or gate capacitance
Explanation: Switching speed and gate drive requirements are dominated by gate charge/gate capacitance (Qg, Cgs, Cgd). These determine how much charge/time the driver needs to change the gate voltage. Body diode forward voltage affects conduction in reverse polarity situations but not primary switching speed.

- Correct answer: d) A positive feedback loop where rising temperature increases leakage/current, causing further heating 
Explanation: Thermal runaway is the destructive positive feedback where temp rise increases leakage/current or lowers resistance, producing more heat and further temperature rise.

- Correct answer: c) Better immunity to external EMI and reduced ground-reference issues 
Explanation: Differential signaling rejects common-mode noise and reduces sensitivity to ground shifts because the receiver senses the difference between two complementary signals.

- Correct answer: b) To route all subsystem grounds to a single central point to minimize ground loops 
Explanation: A star (single-point) ground ties subsystem grounds to one central node to reduce ground loop currents and minimize unwanted interactions between subsystems (useful in mixed-signal systems).

- Correct answer: a) It creates amplitude error/noise proportional to the signal slew rate and jitter magnitude
Explanation: Sampling aperture jitter (timing uncertainty) causes voltage error proportional to instantaneous slope dv/dt of the input: error ≈ (dv/dt)⋅jitter. For high-frequency or fast edges this becomes significant noise/error.

- Correct answer: d) The range of input voltages (relative to rails) over which the inputs behave correctly and the amplifier operates as specified
Explanation: The input common-mode range specifies allowable voltages on the inputs (relative to supply rails) where the op-amp inputs and internal stages remain linear. Driving inputs outside this range can produce incorrect outputs or saturate internal stages.