# DAY6_ELECTRONICS_QUIZE

# Questions

- For switching regulator placement on a PCB, what is the most important reason to place the input bulk capacitor close to the regulator input pin?
a) To minimize the loop area of the input switching current and reduce conducted EMI
b) To reduce the regulator’s gate charge requirements during switching
c) To increase the regulator’s thermal dissipation directly through the cap
d) To lower the board impedance at DC for long-term energy storage

- Which design choice most directly reduces high-frequency radiated EMI from a buck converter switch node?
a) Increasing the bulk capacitance value only
b) Minimizing the switch-node loop area and using a small local decoupling cap
c) Using a larger inductor with higher DC resistance
d) Placing all ground vias far from the switching node

- When designing an anti-aliasing filter for an ADC sampling at fs, which cutoff selection is safest to avoid aliasing of out-of-band noise?
a) fc≈fs
b) fc≈0.9 fs
c) fc≤0.5 fs
d) fc≥2 fs

- In a synchronous buck converter, what is the main advantage of using a synchronous MOSFET instead of a diode for the low-side path?
a) Reduced switching frequency sensitivity only
b) Easier layout with fewer components
c) Lower conduction losses at higher load currents
d) Eliminates need for input filtering entirely

- For precision low-level measurements, which approach most reduces error from sense resistor lead and trace drops on a PCB?
a) Use a Kelvin (four-wire) sense connection and route sense traces directly to resistor terminals
b) Increase the sense resistor value by an order of magnitude to enlarge the voltage drop
c) Place the sense resistor far from the converter to average temperature effects
d) Rely on sampling software to subtract estimated lead drops

- What primary effect does adding series ferrite beads on power rails have for nearby ICs?
a) They improve DC current capacity of the trace significantly
b) They attenuate high-frequency noise by adding impedance at RF frequencies
c) They convert low-frequency ripple into DC offset
d) They reduce ESR of adjacent capacitors

- When paralleling power supplies to share load current, what practice helps achieve better current sharing?
a) Use supplies with widely different output impedances for diversity
b) Match cable lengths only on the negative (return) path
c) Include small ballast resistances (current sharing resistors) in each supply output
d) Disable regulation on one supply so the other masters the voltage

- For lowering component temperature under a power IC, which PCB technique is most effective?
a) Add solder mask over the power pad to insulate it thermally
b) Route thin narrow traces under the IC to slow heat flow
c) Use thermal vias under the top pad tied to internal/bottom copper planes
d) Remove all copper planes to isolate the IC thermally

- When designing a differential pair for high-speed signals, what rule is most important to preserve signal integrity?
a) Keep the two traces as far apart as possible to avoid crosstalk
b) Ensure constant trace spacing and consistent differential impedance along the route
c) Make one trace wider than the other to compensate for return currents
d) Route one trace on top layer and its mate on the bottom with no reference plane

- For EMI troubleshooting, which measurement approach quickly helps identify whether a board emits primarily radiated or conducted noise?
a) Place a power resistor on the rail and measure its temperature rise
b) Use a near-field probe to scan board areas and check the power rail with a LISN for conducted noise
c) Replace all capacitors with larger values and remeasure emissions
d) Increase switching frequency and observe functional failures

# Answers

- Answer: a) — Correct
Explanation: Place the input bulk capacitor close to the regulator input to minimize the input switching-current loop area. That reduces conducted EMI and high-frequency radiation from the switch node.

- Answer: b) — Correct
Explanation: The dominant way to cut high-frequency radiated EMI from a buck switch node is to minimize the switch-node loop area and place a small local decoupling cap right at that node (short traces, proper return path).

- Answer: c) — Correct
Explanation: To avoid aliasing, the anti-alias filter cutoff should be at or below the Nyquist frequency (fs/2). In practice you design the filter so out-of-band noise is attenuated well before fs/2 (often using multi-pole filters or oversampling).

- Answer: c) — Correct
Explanation: In a synchronous buck the low-side MOSFET replaces the diode; when the MOSFET is on it has much lower conduction loss (RDS(on)) than a diode drop, improving efficiency at medium/high loads. It does not eliminate need for input filtering.

- Answer: a) — Correct
Explanation: Kelvin (four-wire) sensing with sense traces routed to resistor terminals minimizes error from lead/trace voltage drops and is the standard PCB practice for precise low-level measurements.

- Answer: b) — Correct
Explanation: Ferrite beads add impedance at RF frequencies and attenuate high-frequency noise on power rails, protecting nearby ICs from conducted/radiated interference. They are not for DC current capacity.

- Answer: c) — Correct
Explanation: For paralleling supplies, small ballast resistors (or intentional output impedance) help equalize output currents and avoid one supply taking most current. Disabling regulation on one supply (d) is not a robust general solution.

- Answer: c) — Correct
Explanation: Thermal vias under a power IC transfer heat to internal or bottom copper planes, significantly improving thermal dissipation and lowering junction temperature—one of the most effective PCB thermal techniques.

- Answer: b) — Correct
Explanation: For differential pairs preserve constant spacing and consistent differential impedance to maintain signal integrity and minimize mode conversion/reflection.

- Answer: b) — Correct
Explanation: Quick EMI diagnosis uses near-field probes to find radiating spots (radiated EMI) and a LISN plus spectrum analyzer to measure conducted emissions on supply rails. That combination distinguishes radiated vs. conducted problems.