# DAY1_ELECTRONICS_QUIZE

# Questions

- What is Ohm's law?
a) V=IR
b) P=I2R
c) Q=CV
d) I=CdVdt

- Which statement is correct about Kirchhoff's laws (KCL and KVL)?
a) KCL refers to the sum of voltages around a loop, KVL refers to the sum of currents at a node
b) KCL states the sum of currents at a node equals zero, KVL states the sum of voltages around a closed loop equals zero
c) KCL and KVL are alternate names for the same law
d) KCL states the sum of powers in a circuit equals the sum of voltages

- What is the general formula to calculate electrical power consumed by a load?
a) P=VI
b) P=V2C
c) P=VI
d) P=I⋅t

- In a voltage divider with two series resistors R1 and R2 connected to an input voltage Vin, what is the expression for Vout measured across R2?
a) Vout=VinR1R1+R2
b) Vout=VinR2R1+R2
c) Vout=VinR1R2R1+R2
d) Vout=Vin1R1+R2

- What is the impedance of a capacitor at angular frequency ω?
a) ZC=jωL
b) ZC=1jωC
c) ZC=R+1jωC
d) ZC=jωC

- How does a capacitor behave when connected to a constant voltage source after the system reaches DC steady state?
a) The capacitor behaves as a short circuit to DC
b) The capacitor behaves as an open circuit to DC
c) The capacitor becomes a pure resistance at DC
d) The capacitor produces a steady current I=C⋅V

- What is the condition for maximum power transfer between a source and a load (impedance matching)?
a) The source resistance must be twice the load resistance
b) The load impedance should equal the source impedance (or the complex conjugate if reactive), e.g. Zload=Zsource*
c) There is no relation between impedance and maximum power transfer
d) Minimizing voltage always maximizes power transfer

- What is the main difference between a buck converter and an LDO (low-dropout regulator)?
a) A buck regulator uses only passive components, an LDO uses an active switching element
b) A buck is a switching converter (high efficiency for large Vin–Vout difference), while an LDO is a linear regulator (generally less efficient with large voltage drop)
c) An LDO is only for high voltages and buck only for low voltages
d) There is no functional difference between them

- What is the primary role of a decoupling (bypass) capacitor placed close to an IC’s power pins?
a) Provide only low-frequency filtering and not affect high-frequency noise
b) Store long-term energy like bulk capacitors
c) Short transient changes in voltage and supply fast local currents to the IC to reduce noise and voltage dips
d) Increase the average voltage on the rail

- What is the purpose of a Kelvin connection (four-wire sensing) when measuring a sense resistor?
a) To measure the resistor voltage via the regular power leads to save wiring
b) To include the lead resistance in the measurement to improve accuracy
c) To isolate the measurement leads from the current-carrying leads and eliminate I⋅R errors from the conductors, yielding an accurate voltage drop measurement across the sense resistor
d) Used only in RF equipment, not in power components

# Answers

- Answer: a) V=IR
Expansion: Ohm’s law states that the voltage V across a resistor is proportional to the current I through it with constant of proportionality R (resistance). Units: V in volts, I in amperes, R in ohms. Related power forms for a purely resistive load: P=VI=I2R=V2R. Example: if R=10 Ω and I=0.2 A then V=IR=2 V and P=I2R=0.4 W. Note: many components (diodes, transistors) are non-linear and do not strictly follow a single R.

- Answer: b) KCL and KVL
Expansion:

- KCL (Kirchhoff’s Current Law): ∑Inode=0. Currents entering a node equal currents leaving. Used for nodal analysis.

- KVL (Kirchhoff’s Voltage Law): ∑Vloop=0. Signed voltage rises and drops around any closed loop sum to zero.
Practical note: keep consistent sign conventions (passive sign convention, loop direction) when writing equations.

- Answer: a) P=VI
Expansion: Instantaneous electrical power is p(t)=v(t) i(t). For DC or steady-state resistive loads P=VI. For resistors substitute V=IR to get P=I2R or P=V2R. For AC with phase shift ϕ between voltage and current, average real power is Pavg=VrmsIrmscos⁡ϕ and complex power S=VrmsIrms*. Note: VI is resistance, not power.

- Answer: b) Vout=VinR2R1+R2
Expansion / derivation: Series current I=VinR1+R2. Voltage across R2 is Vout=I⋅R2=VinR2R1+R2. Practical consideration: connecting a load RL across R2 changes the effective lower resistor to R2∥RL, lowering Vout. Use a buffer (op-amp) or choose divider values so loading is negligible when a stable output is required.

- Answer: b) ZC=1jωC
Expansion: At angular frequency ω=2πf, a capacitor’s complex impedance is ZC=1jωC. Magnitude ∣ZC∣=1ωC and phase -90∘ (current leads voltage by 90∘). Time-domain relation: i=C dvdt. Limits: as ω→0 ∣ZC∣→∞ (open at DC), as ω→∞ ∣ZC∣→0 (short at high frequency). Real capacitors have ESR, ESL and a self-resonant frequency that alter behavior at high frequencies.

- Answer: b) The capacitor behaves as an open circuit to DC
Expansion: After charging to a steady DC voltage, dV/dt=0 so i=C dV/dt=0 (no steady DC current flows through the ideal capacitor). Practical caveats: leakage currents, dielectric absorption and ESR mean a small DC leakage or slow drifts may exist, but functionally the capacitor blocks steady DC current.

- Answer: b) Zload=Zsource* (or RL=Rth for purely resistive source)
Expansion: Maximum power transfer theorem: for Thevenin equivalent Zth=Rth+jXth, real power to the load is maximized when ZL=Zth*=Rth-jXth. For purely resistive case the maximum occurs at RL=Rth and Pmax=Vth24Rth. Important: matching maximizes delivered power but not efficiency — at matched condition efficiency is 50% because half the power is dissipated in the source. Choose matching based on whether you want maximum delivered power or maximum efficiency.

- Answer: b) Buck = switching, LDO = linear
Expansion:

- Buck converter (switching): transfers energy using switches and an inductor; high efficiency (often >90%) especially when Vin-Vout is large. Requires inductor, diode/FET and careful PCB layout to manage switching noise/EMI.

- LDO (linear regulator): regulates by dropping the excess voltage across a pass element; simple and low-noise but dissipates heat Pdiss=(Vin-Vout)⋅Iout. Use LDOs when simplicity, low noise, or low dropout are priorities and power dissipation is acceptable.

- Answer: c) Decoupling caps supply fast local current for transients and reduce noise/dips
Expansion: Decoupling (bypass) capacitors reduce high-frequency impedance of the power rail near an IC so the IC can draw transient current locally. Typical practice: place small ceramic caps (e.g., 0.01–0.1 µF) right at each VCC–GND pin for high-frequency decoupling, and use larger ceramics/tantalums/electrolytics (e.g., 1–100 µF) for lower-frequency bulk energy. Use multiple values to cover a broad frequency range and keep traces/ties very short to minimize loop inductance (ESL).

- Answer: c) Kelvin (four-wire) sensing isolates measurement leads from current leads to eliminate I⋅R lead errors
Expansion: Four-wire sensing uses two force (current) leads to feed current through the sense resistor and two separate sense leads to measure the voltage across the resistor. Because the sense leads draw negligible current, voltage drops in the force leads do not affect the measured voltage, enabling accurate measurement of very low resistances (mΩ or lower). On PCBs implement separate force and sense pads/traces (Kelvin pads) and connect sense traces directly to the resistor terminals to avoid lead-drop errors.