# DAY2_ELECTRONICS_QUIZE

# Questions

- What parasitic element of a discrete resistor becomes significant at high frequencies and can cause the resistor to behave inductively?
a) Equivalent series resistance (ESR)
b) Series inductance (ESL)
c) Dielectric absorption
d) Thermal resistance

- What is a capacitor's self-resonant frequency (SRF)?
a) The frequency where the capacitor's impedance is purely resistive due to ESR
b) The frequency where the capacitor's capacitive reactance equals the inductive reactance of its parasitic inductance, causing resonance
c) The lowest frequency the capacitor can filter
d) The frequency at which the capacitor value doubles

- For a simple series RLC circuit, what is the resonance angular frequency ω0 (ignoring damping)?
a) ω0=1RC 
b) ω0=LC 
c) ω0=1LC 
d) ω0=RL

- How does an ideal inductor behave in the limits of DC and very high frequency?
a) At DC it is an open circuit; at high frequency it is a short circuit
b) At DC it is a short circuit; at high frequency its impedance increases (acts like an open)
c) It is always a pure resistor independent of frequency
d) At DC it stores charge; at high frequency it dissipates energy as heat

- What is the main functional difference between a bypass (decoupling) capacitor and a bulk capacitor on a power rail?
a) Bypass caps provide long-term energy storage; bulk caps filter only RF noise
b) Bypass caps target high-frequency transients and are placed close to IC pins; bulk caps supply low-frequency energy and handle larger charge storage
c) There is no difference; both are identical in value and placement
d) Bulk caps are only used for AC circuits, bypass caps only for DC

- In a PWM (pulse-width modulation) signal, how does changing the duty cycle affect the average voltage delivered to a load (assuming fixed amplitude)?
a) Average voltage is independent of duty cycle
b) Increasing duty cycle increases the average voltage proportionally
c) Increasing duty cycle decreases the average voltage
d) Duty cycle only affects frequency, not average voltage

- What is inductive flyback and why is it important to manage it in circuits with inductive loads?
a) The sudden voltage spike produced when current through an inductor is interrupted; it can damage switches and must be suppressed
b) A steady low-frequency oscillation in inductors that improves efficiency
c) The cooling effect of an inductor under load
d) A method to increase inductance using feedback

- What is a bridge rectifier used for?
a) Converting DC to AC using four diodes
b) Converting AC to DC using a full-wave rectification topology of diodes
c) Matching impedance between source and load
d) Reducing harmonic distortion in PWM signals

- What is a thermal relief pad on a PCB and why is it used?
a) A copper area used to dissipate EMI away from components
b) A pad connected to a plane with thin spokes to make soldering easier and reduce heat conduction into the plane
c) A connection style that increases thermal conduction to speed up cooling during operation
d) A small thermal sensor embedded in the pad

- What is via stitching (via fence) and when is it typically used?
a) Placing a single via under every component to improve mechanical strength
b) A pattern of multiple vias connecting two or more reference planes (e.g., ground) to reduce EMI and improve return current paths, commonly used along edges of high-speed traces or around shields
c) A method of printing vias in different colors for manufacturing visibility
d) A technique to intentionally increase trace impedance by adding vias

# Answers

- answer: b) — Correct
Explanation: At high frequency the resistor’s small series inductance (ESL) dominates and can make the resistor behave inductively. Real resistor model: R in series with ESL and small stray C.

- answer: b) — Correct
Explanation: SRF is where the capacitor’s capacitive reactance equals the inductive reactance of its parasitic ESL, so the part resonates and above SRF it behaves inductively.

- answer: c) (ω0=1/LC) — Correct
Explanation: Undamped resonance angular frequency for ideal LC is ω0=1/LC. (Options like 1/(RC) are RC time constants, not resonance.)

- answer: b) — Correct
Explanation: Ideal inductor impedance is ZL=jωL. At DC (ω=0) ZL=0 (short). As frequency increases ∣ZL∣ increases (high impedance). Real inductors also show parasitic capacitance and SRF at very high frequencies.

- answer: b) — Correct
Explanation: Bypass/decoupling caps handle high-frequency transients at the IC and are placed right at power pins. Bulk caps store larger charge for lower-frequency load changes and are usually larger value and placed near regulators.

- answer: b) — Correct
Explanation: For fixed amplitude PWM, average voltage over a period is proportional to duty cycle. Example: 50% duty on 0–5 V PWM ≈ 2.5 V average (assuming the load or filter averages it).

- answer: a) — Correct
Explanation: Inductive flyback is the large voltage spike produced when current through an inductor is interrupted, from v=L didt. It can damage switches—suppress with diodes (flyback/flywheel diode), snubbers, TVS, RCD clamps, etc.

- answer: b) — Correct
Explanation: Buck converters are switching regulators (efficient for large Vin-Vout) but produce switching noise and require inductors; LDOs are linear, simpler and low-noise but dissipate (Vin-Vout)⋅Iout as heat.

- answer: b) — Correct: b
Explanation: A thermal relief pad is a pad connected to a copper plane by thin spokes (thermal spokes). Purpose: reduce heat conduction into the plane during soldering so the pad heats quickly and solder wets properly. Option c stated increased conduction, which is the opposite.
Practical tip: In your PCB tool, enable thermal relief for through-hole/component pads tied to large copper pours, or manually draw spokes (e.g., 4 spokes, 0.25 mm wide) to the plane.

- answer: b) —  Correct: b
Explanation: Via stitching (via fence) is a pattern of multiple vias connecting reference planes (usually ground) to reduce EMI, provide low-impedance return paths, and improve shielding (commonly placed along plane seams, board edges, or around differential pairs/shields). A single via under each component does not achieve the same effect.
Practical tip: For ground stitching, place vias at regular intervals (e.g., 1–3 mm for very high-speed/EMI-critical edges, 5–10 mm for general use) and tie them to the ground plane(s); for split planes, stitch along plane boundaries to keep return currents local.